| S.No | Test Point | Measured Voltage (V) | Remarks |
| --- | --- | --- | --- |
| 1 | Battery Supply | 8.58V | Expected ≈ 9 V |
| 2 | LM358 Pin 8 (VCC) | 8.20V | Supply voltage |
| 3 | LM358 Pin 4 (GND) | 0V | Expected ≈ 0 V |
| 4 | LM358 Pin 2 (LDR Input) - Bright | 7.70V | Voltage with light on LDR(LED OFF) |
| 5 | LM358 Pin 2 (LDR Input) - Dark | 5.5V | Voltage with LDR covered(LED ON) |
| 6 | LM358 Pin 3 (Reference from Potentiometer) | ________ | Changes when preset is rotated |
| 7 | LM358 Pin 1 (Output) - Bright | 0.05V | Comparator output(LED OFF) |
| 8 | LM358 Pin 1 (Output) - Dark | 6.69V | Comparator output(LED ON) |
| 9 | BC547 Base | 0.72V | Around 0.6–0.7 V when ON |
| 10 | LED Voltage | 2.88V | Voltage across LED |